Table of Contents

OVERVIEW	3

HISTORY	3

Timeline:	3

INTERNET OF THINGS	4

Low-power wide-area network (LPWAN)	4

The Things Network	5

HELIUM ECOSYSTEM	6

Network architecture	7

Wireless protocol (LoRa)	7

LoRaWAN	8

LoRa Alliance	8

Blockchain	8

Components and concepts	8

Helium Token (HNT)	9

Data Credits (DC)	10

Proof of coverage	10

Proof of Coverage Challenges	10

Consensus Protocol	11

Mining and Token reword	11

DEVICES/USERS	13

Miners types	13

Light Hotspot	13

Data only Hotspot	13

Full Hotspot	13

Validators	13

Manufacrurers	14

Profit	14

SECURITY RISKS	15

Helium related	15

Local network	15

Exposed device location	15

Too many manufacturers/Uncertified devices	15

Risks from IoT devices	15

Botnet	16

Drones control	16

Real-time field data for military use	16

Attacks on IoT	16

Denial of service	17

Man-in-the-middle	17

Advanced persistent threats	17

Signal jamming	18

APPENDIX A	19

APPENDIX B	20

APPENDIX C	21

APPENDIX D	34

# OVERVIEW

Helium is an Internet-of-Things developer platform for transferring IoT device data. It provides a decentralized open wireless network built on a new blockchain for the physical world (also known as The People’s Network). It relies on a novel type of work called Proof of Coverage, and a new consensus algorithm (based on HoneyBadger BFT). The Helium Network is used to route data for long-range, lower power IoT devices that use LoRaWAN (previously Helium's LongFi protocol).

- Location: San Francisco, California, United States

- Founded Date: 2013

- Number of Employees: 30 (APPENDIX A)

- Legal Structure: Corporation

- Founders: Amir Haleem, Shawn Fanning and Sean Carey

Keywords: Blockchain, Cryptocurrency, Internet of Things, Telecommunications, Wireless, LoRaWAN.

# HISTORY

Founded in 2013 with the idea to create a global network for IoT devices. In December 2014, the company announced that it had raised $16 million in funding led by Khosla Ventures, with participation from other investors (APPENDIX B).

They began by building a proprietary radio protocol embedded in sensors. The approach did not find a good success, so Helium changed up the model.

Inspired by the Filecoin (FIL) white paper in 2017, they thought about building the network based on blockchain that relies on “proof of coverage” instead of “proof-of-work”, the process Bitcoin’s miners use.

## Timeline:

- May 2018: whitepaper

- June 2019: Helium revealed its first physical Helium hotspot devices, which would power its peer-to-peer IoT infrastructure and using LongFi wireless protocol.

- July 2019: Helium blockchain Main net goes live

- August 2019: The first city-wide Helium network is deployed in Austin, TX

- October 2019: Helium released most components of the network as open-source software and hardware. This includes the Helium blockchain, the LongFi wireless protocol, various hardware schematics, developer client libraries, hardware schematics, and cloud APIs.

- March 2020: Swapping LongFi wireless protocol for LoRaWAN in collaboration with Semtech.

- December 2020: manufacturers can also apply with the Helium community to mass-produce hotspots of their own, and more than 13 companies have successfully done so to date.

# INTERNET OF THINGS

In the broadest sense, the term IoT encompasses everything connected to the internet, but it is increasingly being used to define objects that "talk" to each other. "Simply, the Internet of Things is made up of devices – from simple sensors to smartphones and wearables – connected together".

By combining these connected devices with automated systems, it is possible to "gather information, analyze it and create an action" to help someone with a particular task, or learn from a process. In reality, this ranges from smart mirrors to beacons in shops and beyond.

## Low-power wide-area network (LPWAN)

A low-power wide-area network (LPWAN) is a type of wireless telecommunication wide area network designed to allow long-range communications at a low bit rate among things (connected objects), such as sensors operated on a batter. The low power, low bit rate, and intended use distinguish this type of network from a wireless WAN (Wi-Fi, Bluetooth...) that is designed to connect users or businesses, and carry more data, using more power.

A LPWAN may be used to create a private wireless sensor network but may also be a service or infrastructure offered by a third party, allowing the owners of sensors to deploy them in the field without investing in gateway technology.

The cellular industry is trying to build a low-cost network for IoT on traditional licensed spectrum called NB-IoT, while sticking to the traditional centralized deployment, maintenance, and pricing approach where the network is operated by few companies. Long Established wireless carriers didn’t see a business case for building networks geared for devices that might transmit less than a dollar’s worth of data per year, what opened the field for startups to find new solutions.

Sigfox, Senet and LoRa alliance are also trying to build low-power IoT networks from scratch. The Things Network which uses LoRaWAN was getting popular in 2015 followed by Helium in 2019.

## The Things Network

The Things Network (TTN) is an open community-based initiative to establish an energy-saving wide-area network, as for the Internet of Things. It was started in 2015 by the two Dutchmen Wienke Giezeman and Johan Stooking, and it is a part of the LoRa Alliance.

Software developers from The Things Network develop open source LoRaWAN solutions and offer open programming tools and a global community to create Internet of Things (IoT) applications and to build a collaborative Internet of Things network that spans many countries around the world. The Network is a global, open, free, and decentralized crowdsourcing project. Volunteers take on the provision, construction, and support of LoRaWAN gateways.

In this way, Amsterdam has succeeded in covering large parts of the urban area in less than six weeks. Most of the Netherlands are already supplied with TTN. The same applies to Zurich, Bern, and Berlin. In Berlin, over 210 registered gateways currently provide around 1,500,000 people with LoRaWAN access.

In mid-February 2021, around 18,000 LoRaWAN gateways were in operation in around 150 countries and around 135,000 volunteers around the world were involved in setting up the world's largest, networked "Internet of Things", in which millions of data records are processed every day by millions of people supply.

Figure 1 TTN architecture

HELIUM ECOSYSTEM

Helium Network is hosted by Helium blockchain, which supports the decentralized Network. Decentralized means that hotspots are operated and hosted by normal users who in exchange earn a passive income with Helium token (HNT). The People’s Network allows any LoRa device (also called end devices, machines, nodes, sensors, objects, things …) to connect to the network through Helium Hotspots. Moreover, businesses using Helium Network as end users need to only pay for their actual data usage. The cost of running a sensor on the Helium blockchain can be just cents per year.

Helium provides a service called Console, which is a web-based device management tool that allows developers to register, authenticate, and manage their devices on the Helium network. In addition to device management, Console provides prebuilt connections called Integrations to route device data via HTTPs or MQTT; or directly to cloud services like AWS IoT.

Therefore, 2 main categories of users benefit from the network:

- Miners, they are hotspot operators who will get rewarded with HNT.

- The IoT users benefitting from the LoRa coverage, allowing them to setup their sensors and easily route the data flow with a secure connection.

## Network architecture

The Helium network provides internet connection for devices that take a form of hardware embedded with a LoRa chip and a compatible firmware. Devices spend tokens by paying miners to send data to and from the internet. The network is a LPWAN solution where end devices are mostly battery powered, and solutions such as Wi-Fi/4G/5G does not fit. The helium routers allow data to be forwarded to private databases, network monitoring system or hosting services.

On the other hand, miners participate in the creation of new blocks in the blockchain. They earn tokens (HNT) by providing wireless network coverage via purpose-built hardware (hotspots) which provide a bridge between end devices and routers. Miners join the network by asserting their satellite-derived location, a special type of transaction in the blockchain, and staking a token deposit. Miners are paid once they prove they have delivered data to the specified router.

Helium Console provides a web browser front-end for developers to provision and manage devices, monitor data activity, and manage connectivity costs. For flexibility, the Console is built on an extensible set of REST APIs.

## Wireless protocol (LoRa)

LoRa is a proprietary LPWAN modulation technique. It is based on spread-spectrum modulation techniques derived from chirp spread spectrum (CSS) technology. It was developed by Cycleo of Grenoble, France and acquired by Semtech, the founding member of the LoRa Alliance.

LoRa uses license-free sub-gigahertz radio frequency bands like EU433 (433.05-434.79 MHz) and EU863-870 (863–870/873 MHz) in Europe; AU915-928/AS923-1 (915–928 MHz) in Australia; US902-928 (902–928 MHz) in North America; IN865-867 (865–867 MHz) in India; AU915-928/AS923-1 and EU433 Southeast Asia. (See Appendix C)

LoRa enables long-range transmissions with low power consumption. The technology covers the physical layer, while other technologies and protocols such as LoRaWAN (Long Range Wide Area Network) cover the upper layers. It can achieve data rates between 0.3 kbit/s and 27 kbit/s, depending upon the spreading factor.

LoRa devices have geolocation capabilities used for trilaterating positions of devices via timestamps from gateways.

### LoRaWAN

LoRaWAN is a cloud-based medium access control (MAC) layer protocol but acts mainly as a network layer protocol for managing communication between LPWAN gateways and end-node devices as a routing protocol, maintained by the LoRa Alliance.

LoRaWAN defines the communication protocol and system architecture for the network, while the LoRa physical layer enables the long-range communication link. LoRaWAN is also responsible for managing the communication frequencies, data rate, and power for all devices. Devices in the network are asynchronous and transmit when they have data available to send. Data transmitted by an end-node device are received by multiple gateways, which forward the data packets to a centralized network server. Data are then forwarded to application servers. The technology shows high reliability for the moderate load.

### LoRa Alliance

The LoRa Alliance is a 501(c)(6) association created in 2015 to support LoRaWAN protocol, as well as to ensure interoperability of all LoRaWAN products and technologies. This open, nonprofit association has over 500 members. Some members of the LoRa Alliance are IBM, Everynet, Actility, MicroChip, Orange, Cisco, KPN, Swisscom, Semtech, A2A Smart City SPA, Bouygues Telecom, Singtel, Proximus, The Things Industries and Cavagna Group.

The Alliance is administered by the VTM Group in Beaverton, Oregon.

## Blockchain

The Helium Blockchain is based on a new, novel work algorithm called Proof of Coverage, and rewards miners in $HNT, the native token of the Helium blockchain. Officially launched on July 29, 2019.

### Components and concepts

Blocks:

Blocks are used to record the most recent set of transactions in the Helium blockchain. Blocks are mined based on time, as defined in the block_time chain variable. The current target block time is 60000 milliseconds (or 60 seconds). During any given epoch, the most recent block consists of:

- Block Version

- Block Height

- Previous Block Hash

- Transactions (stored as a Merkle hash)

- Threshold signature from the current consensus group

Epochs:

An epoch is the target time period for which a given group of Miners is elected to serve as the consensus group. The target time for an epoch is currently 30 blocks, as defined in the election_interval chain variable. Approximately every 30 blocks mined marks the passing of an epoch, after which a new group of Miners is elected to form the next consensus group. Mining rewards are distributed per epoch (as opposed to per block in most blockchain-based systems). At the conclusion of each epoch, the consensus group will distribute all the $HNT produced in that block via the rewards transaction.

#### Accounts, Wallets, and Keys:

The Helium blockchain uses an account-based system for wallets and balances (similar to Ethereum and others) as opposed to a UTXO-based ledger like Bitcoin blockchain. Users have several options for creating Helium wallets and managing their private keys.

Both the Helium Mobile Wallet and the CLI Wallet create and use keys based on the ed25519 public key signature system. The CLI can also support NIST p-256 keys. And the Helium blockchain supports both key types.

### Helium Token (HNT)

The Helium Token is the native cryptocurrency and protocol token of the Helium blockchain. Its symbol is HNT. Prior to the launch of the Helium blockchain, there was no premine of HNT. The first HNT was produced on July 29th, 2019 as part of the block 93.

The Helium Token is designed to serve the needs of the two primary parties in the Helium blockchain ecosystem:

- Hotspot Hosts and Network Operators. Hosts mine HNT while deploying and maintaining network coverage.

- Enterprises and developers using the Helium Network to connect devices and build IoT applications. Data Credits, which are a $USD-pegged utility token derived from HNT in a burn transaction, are used to pay transaction fees for wireless data transmissions on the network (in addition to things like adding Hotspots and sending).

The Helium blockchain uses three distinct token economic concepts to ensure HNT supply is both plentiful for network needs but also relatively scarce, with a known maximum.

At launch genesis block, the Helium Network targeted 5,000,000 HNT per month. And since the community approval of HIP 20, the Helium blockchain uses a two year halving schedule, with a maximum supply of 223,000,000 HNT.

### Data Credits (DC)

Data Credits are a $USD-pegged utility token derived from HNT in a burn transaction and used to pay all transaction fees on the Helium Network. One Data Credit will always cost $0.00001. Or, $1.00 will always buy you 100,000 Data Credits. But, as noted above, Data Credits are produced by burning HNT. And the market price of HNT will of course fluctuate.

This HNT to DC relationship is based on a design commonly called a burn and mint equilibrium and is intended to allow for the supply of HNT to respond to network usage trends such that, when equilibrium is found, the amount of HNT that exists remains static month on month. (Helium borrowed heavily from Factom’s usage of this design.) The amount of Data Credits produced by burning HNT will move up and down based on the USD price of HNT as reported by the HNT Oracles.

### Proof of coverage

Proof of Coverage is the new algorithm that was created for Helium. It verifies that the Hotspots in the network are physically located where they claim, and that they are honestly representing the wireless coverage that’s being created by the Hotspot for its location.

Proof of Coverage relies on these three characteristics:

- RF has limited physical propagation and, therefore, distance.

- The strength of a received RF signal is inversely proportional to the square of the distance from the transmitter; and

- RF travels at the speed of light with (effectively) no latency.

Through these properties the blockchain is always using the PoC challenge mechanism to interrogate Hotspots regarding their location and coverage. This allows the Helium Network to constantly use the data generated to definitively verify the exact wireless coverage that’s provided by the network’s Hotspots.

### Proof of Coverage Challenges

The PoC challenge is the discrete unit of work used by the Proof of Coverage algorithm. In the less than two years since the Helium network has been launched there have been tens of millions of challenges issued and processed by the Helium blockchain. As each new challenge is issued and processed the Helium blockchain receives and records more information regarding the quality of the network.

The PoC challenges all involve three distinct roles on the network:

- Challenger – The Hotspot that constructs and issues the POC Challenge. Hotspots issue challenges approximately once every 240 blocks.

- Transmitter – Also called the “Challengee”, this Hotspot is the target of the POC challenge and is responsible for transmitting (or “beaconing”) challenge packets to potentially be witnessed by other geographically proximate Hotspots.

- Witness – Hotspots that are geographically proximate to the Transmitter and report the existence of the challenge packet after it has been transmitted.

### Consensus Protocol

Consensus in Helium is based on a variant of the HoneyBadger BFT protocol.

HoneyBadger BFT was created as an asynchronous atomic broadcast protocol that enables a collection of known nodes to achieve consensus over a set of unreliable links. Helium has implemented the consensus in such a way that the consensus group of elected Hotspots receives encrypted transactions and then work to reach an agreement on how these transactions should be ordered. They then add the transactions to a new block and add that block to the blockchain.

The scheme that allows HoneyBadger BFT to work properly is called threshold encryption. With this all transactions are encrypted with a shared public key. The transactions can only be decrypted when the entire elected consensus group works together to decrypt them. By using this scheme Helium is able to achieve censorship-resistant transactions.

A new Consensus Group (CG) is elected once per epoch. Currently there are 40 members elected to each consensus group. At the end of each epoch, mining rewards are distributed by the consensus group to the wallet addresses that have earned them. Currently 65% of all mining rewards go to the hotspot infrastructure (with the remaining 35% being distributed to Helium, Inc and other network investors).

### Mining and Token reword

The Helium blockchain rewards Hotspots for providing wireless coverage and verifying the Helium Network. Hotspots are rewarded in Helium Token, $HNT.

| Reward Type | Description |
| --- | --- |
| PoC Challenger | Rewarded to any Hotspot that creates a valid PoC challenge and submits the corresponding receipt to the blockchain. |
| PoC Challengees | Awarded to any Hotspot that transmits a POC packet after being targeted by the challenger. |
| Witnesses | Distributed to all Hotspots that witness a beacon packet as part of a PoC Challenge. |
| Consensus Group | Divided equally among the Validators that are part of the outgoing Consensus Group, responsible for mining blocks. |
| Security | Awarded to Helium, Inc and other Network investors who hold Security Tokens. |
| Network Data Transfer | Distributed each epoch to Hotspots that route LongFi sensor data for sensors on the Network during that epoch. |

Every epoch, the current consensus group mines approximately 30 blocks on the blockchain. In each block, Hotspots perform various types of work and are awarded according to the following distribution:

| Reward Type | Percentage |
| --- | --- |
| PoC Challenger | 0.90% |
| PoC Challengees | 5.02% |
| Witnesses | 20.08% |
| Consensus Group | 6% |
| Security Tokens | 33% |
| Network Data Transfer | Up to 35% |
| Total | 100% |

Below are the mining rewards per epoch as of August 1, 2021. For every complete epoch, marked by the election of a new Consensus Group, all the HNT produced are distributed over the following reward types:

# DEVICES/USERS

## Miners types

### Light Hotspot

Light Hotspots are the evolution of how LoRaWAN gateways will interact with the Helium blockchain. Since the genesis block, the Helium blockchain has run entirely on the physical Hotspot itself. As the blockchain network grows, each Hotspot running a full node undergoes increasing load and are starting to fall out of sync due to the higher compute requirements.

To mitigate this, the Hotspot's consensus work has been moved off of physical Hotspots and onto Validators as of July 8, 2021. This simplifies the architecture and enables us to grow the Network exponentially while keeping blocktimes and epochs stable.

As all approved Hotspots transition to a Light Hotspot architecture, we also introduce a new type of Hotspot known as Data Only Hotspots, activated as of July 20, 2021, to satisfy the increasing demand and continue to expand network coverage.

As of October 11 2021, Data-Only Hotspots can mine HNT for forwarding data packets on the network.

### Data only Hotspot

Data Only Hotspots are eligible for Network Data Transfer Rewards but not Proof of Coverage Rewards.

Since Data Only Hotspots do not earn Proof-of-Coverage rewards, they may be permissionlessly added to the blockchain. Once added to the blockchain and with location asserted, they will earn $HNT when forwarding network data.

### Full Hotspot

Full Hotspots are Helium coverage providers that can participate in all potential reward activity available to Hotspots. Compared to a Data Only Hotspots, Full Hotspots are eligible for Proof-of-Coverage Rewards.

Full Hotspots may only be manufactured by approved Makers. These are hardware vendors that have been approved by the Helium community and the DeWi. This process is governed via Helium Improvement Proposal 19 (HIP19).

### Validators

Validators are a new entity on the Helium Blockchain (as of July 8 2021) that perform the work of the consensus group including verifying transactions and adding new blocks to the blockchain. They are run on a virtual server on a cloud service like AWS, GCP, Azure, or Digital Ocean. Or a physical server that is deployed at home or data center.

Proof of stake validators will verify hotspots’ transactions, perform consensus, and add new blocks to the Helium blockchain, earning rewards in HNT for contributing to the network’s stability. Each Helium validator must have 10,000 HNT staked to it in order to participate. A minimum of 100 validators has been proposed for the active set; once that threshold has been reached, the protocol will begin electing validators from the pool to perform the work of the consensus group. There is no maximum to the active set, incentivizing joining the network as a validator early while the pool to be selected to perform work is lower.

Each epoch (≈30 blocks), the protocol uses a probabilistic beacon to select a number of validators (estimated from a few dozen to the low hundreds) from the total validator pool to form the Consensus Group. Unlike most PoS networks, validators are selected to participate semi-randomly rather than based on the amount of HNT staked. As such, operators optimize their participation in the network by operating as many validator nodes as possible.

## Manufacrurers

Helium, Inc. was the first Hotspot manufacturer. Starting in early 2019, they produced the first hardware capable of mining HNT and providing LoRaWAN wireless coverage. In total, Helium, Inc. produced and sold the first 13,000 Hotspots to the Helium community to seed the Network.

Since the introduction of HIP19, additional approved Makers have been added. Each approved Maker is given a unique onboarding address on the Helium blockchain where they are required to burn data credits to onboard their unique Hotspots before they are granted access to the blockchain. (for list of manufacturers see APPENDIX D).

## Profit

Your rewards come from a mixture of factors.

Quantity of miners you are able to see. Height and line of sight are key here for maximum distance and rf penetration. If there are no miners within a reasonable distance from yours, you wont witness anyone. Also your placement is only half of the equation when it comes to making a connection. If the other miner is in a basement for instance, it may not matter how close you are to it, they wont communicate. Which brings me to #2.

Quality of miners you are able to see. Your rewards for witnessing are directly affected by the beaconing hotspot's transmit scale. Meaning, if they have a 1.00 scale you will receive the full reward for witnessing that beacon. .5 = half the full reward and so on. This is determined by saturation of that hotspot in not only the individual (res 8) hex, but also res 7,6,5,4 hexes that are even larger. (Check out hotspotty or helium.vision for these hexes).

Additionally, the max number of witness' per beacon is currently 18. Meaning, if 19 or more hotspots witness a beacon, only 18 are randomly chosen to witness said beacon. Also, the more hotspots that witness the same beacon as you, the more the rewards are divided. ( Example: you are 1 of 4 hs that witness a beacon = larger reward. You are 1 of 18 hs that witness a beacon = small reward).

Combined with the transmit scale, you now have a rough understanding of rewards distribution.

3. External conditions. Rf can be wonky and so can the network as it grows. Earnings will not be consistent day to day. Patience is king

# SECURITY RISKS

## Helium related

These are risks that can be a result of the hotspot device itself when deployed and maintained by average users.

### Local network

The Helium hotspot is connected to the local network, and this inherits security flows like any wi-fi device nowadays, as other computers and devices might be exposed if a malware or a virus is spread on the LAN or the hotspot device is compromised/hacked.

### Exposed device location

As seen previously, each Helium device must publicly announce its location and it is regularly checked. This allows the public to know the exact address where the hotspot is located, and it can be an issue for some users especially in areas with very few hotspots or if the coverage is challenged by other users to monopolize the profit.

### Too many manufacturers/Uncertified devices

The system is open source and 3rd party manufacturers and coders are allowed to make their own devices. This introduces the risk of devices being altered or having additional hidden functionalities that are not declared. (RF jamming, signal Tx/Rx for another purpose). Also, the high number of manufacturers and series will hardly keep them under check for malicious activity.

## Risks from IoT devices

IoT in general can be taken advantage of, by military and security parties either for offence or defense. The way military can deploy IoT devices can be either with the help of agents, or a cluster deployment during battle. The use case is open, and it is not limited to the cases specified below.

### Botnet (TCP/IP devices)

A botnet is a network that combines various systems together to remotely take control over a victim’s system and distribute malware. Cybercriminals control botnets using Command-and-Control-Servers to steal confidential data, acquire online-banking data, and execute cyber-attacks like DDoS and phishing. Cybercriminals can utilize botnets to attack IoT devices that are connected to several other devices such as laptops, desktops, and smartphones.

Mirai botnet has displayed how dangerous IoT security threats can be. The Mirai botnet has infected an estimated 2.5 million devices, including routers, printers, and smart cameras. Attackers used the botnet to launch distributed denial of service attacks on several IoT devices. After witnessing the impact of Mirai, several cybercriminals have developed multiple advanced IoT botnets.

### Communications

The established LoRa network can be used as a communication route on top of other existing mobile, internet and radio networks but with the potential to cover wider areas and using third party and unsecured devices.

### Tracking

Using GNSS equipped devices the network can be used to periodically (or asynchronously) stream data, showing the device location. Again the wide coverage will make the system less expensive to operate in deserted areas.

### Drones control

The LoRa network can be used as an alternative path to control drones from. In “Beyond Line of Sight” operations (BVLOS), drone operators look for other means to reach the drone location in order to control it remotely. The existence of such a large and redundant network can facilitate these kind of operations, simply by treating the drone as an IoT device.

### Real-time field data for military use

These are examples of IoT systems that can be used by the military beyond their borders for defensive purposes.

#### Data for training/Simulations

The field keeps changing and it is hard to keep track of all changes in hostile environment. IoT can help the military to assess some places of interest in real-time, and conduct training for its personnel on the recent changes. The data can be also beneficial for target recognition AI.

#### Movement/audio/signal detection

IoT sensors are very useful in wide and empty areas for detecting movements or voices, then locating the source using sophisticated algorithms. This allows the operator to detect any kind of activities in places that needs to be secure like borders and strategic points, or also uncovering secret bases.

#### Iron Dome early warning

Another example of the potential of IoT is the use in early warning as the case in Iron Dome system. The sensors can recognize the event of missile launch or even predict the trajectory very early, relying on data such as air pressure and temperature, or cameras with edge AI.

## Use cases

### Disaster management

#### Mitigation

IoT devices and sensors can collect near-real-time data on things like water levels, volcanic activity, and barometric readings. Sensors can detect wildfires, tornadoes, cloudbursts, volcanic activities, earthquakes, etc. and send early warnings. Additionally, critical infrastructure can be protected through predictive maintenance. Hazard mitigation is enabled by using sensors to monitor pollutants and contaminants, including radioactive situations.

#### Preparation

Emergency preparedness can be enhanced through IoT-enabled response mechanisms, procedures and rehearsals. Real-time data from sensors, cameras, and other connected devices can be embedded into infrastructure, making it possible to monitor conditions in real time, as well as receive vital data both historical and in real time. Additionally, through the use of IoT devices, strategic reserves of food, water, clothing, medical equipment, and other vital supplies can be monitored to ensure acceptable levels.

#### Response

In the immediate aftermath of a disaster, situational awareness is critical to ensuring resources are prioritized in order to have the most impact and help those most in need, but as operations go on, recovery efforts must be repeatedly updated based on changing conditions. Widely deployed IoT technology within a city’s infrastructure, in forest areas, or anywhere else, for that matter, can be re-tasked to identify emergency conditions, people who are trapped, or the status of things like the power grid. First responders need actionable information and IoT can facilitate response planning and actions through the use of sensors to monitor the movement of key personnel, as well as sensors and IoT-enabled cameras on the scene of the incident. Situational awareness and incident management can be achieved through things like smart clothing, which can monitor and report things like a firefighter’s vital signs and on-scene conditions, allowing them to be pulled from the scene if things become too dangerous. First responders can also be equipped with audio and video sensors, or supported by autonomous drones and vehicles, allowing dangerous situations to be monitored and assessed from a safe distance.

### Intrusion detection

IoT like many other technologies that are deployed can be useful for intrusion detection systems. Widespread collected data provide the real-time monitoring and awareness of the situation at a given location, which can facilitate the detection of unwanted presence of activities.

### Valuables condition monitoring

Monitoring of valuables in warehouses, industries, museums, etc… is very important to protect from thievery or deterioration. Using IoT sensors instead of human monitoring can reduce the cost as well as providing the ability to predict potential dangers to the materials in question.

### Tracking

The large coverage of the Helium network allows LoRa IoT devices to be connected in places that are not covered by cellular networks, and thus, tracking devices equipped with GNSS systems will be online in more areas throughout the country.

### Communication

As said before the Helium network can be used as a new untraditional communication channel, any 3rd party device can be enabled to use as a communication device unrestricted by the government and large telecom companies regulations.

### Civilian and domestic use

A LoRa network with large coverage will enable all sort of civilian IoT applications as well as research to enhance the daily life. Examples of such applications could be: smart irrigation, water monitoring, iot medical devices, smart home, smart city, traffic control public transport, etc…

## IoT Challenges

Like every new technology, IoT potentially introduces a new attack surface. This attack surface consists of:

- IoT devices

- Communication channels between the devices as well as between the devices and the back-end system

- IoT-specific back-end applications

- Back-end data storage.

Below are common attacks against IoT.

### Denial of service

A denial-of-service (DoS) attack deliberately tries to cause a capacity overload in the target system by sending multiple requests. Unlike phishing and brute-force attacks, attackers who implement denial-of-service don’t aim to steal critical data. However, DoS can be used to slow down or disable a service to hurt the reputation of a business. For instance, an airline that is attacked using denial-of-service will be unable to process requests for booking a new ticket, checking flight status, and canceling a ticket. In such instances, customers may switch to other airlines for air travel. Similarly, IoT security threats such as denial-of-service attacks can ruin the reputation of businesses and affect their revenue.

### Man-in-the-middle

In a Man-in-the-Middle (MiTM) attack, a hacker breaches the communication channel between two individual systems in an attempt to intercept messages among them. Attackers gain control over their communication and send illegitimate messages to participating systems. Such attacks can be used to hack IoT devices such as smart refrigerators and autonomous vehicles.

Man-in-the-middle attacks can be used to attack several IoT devices as they share data in real-time. With MiTM, attackers can intercept communications between multiple IoT devices, leading to critical malfunction. For instance, smart home accessories such as bulbs can be controlled by an attacker using MiTM to change its color or turn it on and off. Such attacks can lead to disastrous consequences for IoT devices such as industrial equipment and medical devices.

### Advanced persistent threats

Advanced persistent threats (APTs) are a major security concern for various organizations. An advanced persistent threat is a targeted cyber attack, where an intruder gains illegal access to a network and stays undetected for a prolonged period of time. Attackers aim to monitor network activity and steal crucial data using advanced persistent threats. Such cyber attacks are difficult to prevent, detect, or mitigate.

With the advent of IoT, large volumes of critical data are effortlessly transferred among several devices. A cybercriminal can target these IoT devices to gain access to personal or corporate networks. With this approach, cybercriminals can steal confidential information.

### Malwares(TCP/IP devices)

Malwares are malicious processes spread by hackers to take advantage or sabotage devices. The target of these malware is usually devices that work on TCP/IP protocol and that excludes the LoRa devices but can apply to IoT in general once it is reverse-engineered.

### Signal jamming

The IoT are usually connected in a star topology meaning that a large number of nodes are connected to one hotspot that is connected to the internet. This configuration creates a single point of failure when the hotspot is exposed to intensive interference or jamming.

# APPENDIX A

List of employees in Helium inc.

- Abhay Kumar

Chief Product Office

- Amir Haleem

CEO & Co-Founder

- Frank Mong

Chief Operating Officer

- Marc Nijdam

CTO

- Noor Fateh

Founder and Chief Executive Officer

- Andrew Allen

VP of Engineering

- Chris Bruce

Vice President Product and Technology

- Dal Gemmell

Head of Product Marketing and Planning

- Woody Schneider

VP Sales

- Coco Tang

Director of Product

More employees and related people:

https://www.linkedin.com/company/heliuminc/people/

# APPENDIX B

List of investors

FirstMark Capital,

Digital Garage,

Marc Benioff,

SV Angel,

Google Ventures,

HSB/MunichRe Ventures and others

# APPENDIX C

LoRa allocated frequency bands per country

#### A

| Country | Frequency Plan |
| --- | --- |
| Afghanistan |  |
| Albania | EU868 |
| Algeria | AS923_3 |
| American Samoa | US915 |
| Andorra | EU868 |
| Angola |  |
| Anguilla | AU915 |
| Antarctica |  |
| Antigua and Barbuda |  |
| Argentina | AU915 |
| Armenia | EU868 |
| Aruba |  |
| Australia | AU915 |
| Austria | EU868 |
| Azerbaijan | EU433 |

#### B

| Country | Frequency Plan |
| --- | --- |
| Bahamas | US915 |
| Bahrain | EU868 |
| Bangladesh | AS923_1 |
| Barbados | AU915 |
| Belarus | EU868 |
| Belgium | EU868 |
| Belize | AU915 |
| Benin | EU868 |
| Bermuda | US915 |
| Bhutan | EU868 |
| Bolivia | AU915 |
| Bosnia and Herzegovina | EU868 |
| Botswana | EU868 |
| Bouvet Island | EU868 |
| Brazil | AU915 |
| British Indian Ocean Territory |  |
| British Virgin Islands | AU915 |
| Brunei | AS923_1 |
| Bulgaria | EU868 |
| Burkina Faso |  |
| Burundi | EU868 |

#### C

| Country | Frequency Plan |
| --- | --- |
| Cambodia | AS923_1 |
| Cameroon | EU433 |
| Canada | US915 |
| Cape Verde |  |
| Cayman Islands | US915 |
| Central African Republic |  |
| Chad |  |
| Chile | AU915 |
| China | CN470 |
| Christmas Island | AU915 |
| Cocos [Keeling] Islands | AU915 |
| Colombia | AU915 |
| Comoros | EU868 |
| Congo [DRC] |  |
| Congo [Republic] |  |
| Cook Islands | AU915 |
| Costa Rica | AS923_1 |
| Côte d'Ivoire | EU868 |
| Croatia | EU868 |
| Cuba | AS923_3 |
| Cyprus | EU868 |
| Czech Republic | EU868 |

#### D

| Country | Frequency Plan |
| --- | --- |
| Denmark | EU868 |
| Djibouti |  |
| Dominica | AU915 |
| Dominican Republic | AU915 |

#### E

| Country | Frequency Plan |
| --- | --- |
| Ecuador | AU915 |
| Egypt | EU868 |
| El Salvador | AU915 |
| Equatorial Guinea | EU868 |
| Eritrea |  |
| Estonia | EU868 |
| Eswatini [Swaziland] |  |
| Ethiopia |  |

#### F

| Country | Frequency Plan |
| --- | --- |
| Falkland Islands [Islas Malvinas] | EU868 |
| Faroe Islands | EU868 |
| Fiji |  |
| Finland | EU868 |
| France | EU868 |
| French Guiana | EU868 |
| French Polynesia | EU868 |
| French Southern Territories | EU868 |

#### G

| Country | Frequency Plan |
| --- | --- |
| Gabon |  |
| Gambia | EU433 |
| Gaza Strip |  |
| Georgia | EU868 |
| Germany | EU868 |
| Ghana | EU433 |
| Gibraltar | EU868 |
| Greece | EU868 |
| Greenland | EU868 |
| Grenada | AU915 |
| Guadeloupe | EU868 |
| Guam | US915 |
| Guatemala | AU915 |
| Guernsey | EU868 |
| Guinea | EU433 |
| Guinea-Bissau |  |
| Guyana | US915 |

#### H

| Country | Frequency Plan |
| --- | --- |
| Haiti |  |
| Heard Island and McDonald Islands | AU915 |
| Honduras | AU915 |
| Hong Kong | AS923_1 |
| Hungary | EU868 |

#### I

| Country | Frequency Plan |
| --- | --- |
| Iceland | EU868 |
| India | IN865 |
| Indonesia | AS923_2 |
| Iran | EU868 |
| Iraq |  |
| Ireland | EU868 |
| Isle of Man | EU868 |
| Israel | AS923_4 |
| Italy | EU868 |

#### J

| Country | Frequency Plan |
| --- | --- |
| Jamaica | AU915 |
| Japan | AS923_1 |
| Jersey | EU868 |
| Jordan | IN865 |

#### K

| Country | Frequency Plan |
| --- | --- |
| Kazakhstan | EU433 |
| Kenya | EU868 |
| Kiribati |  |
| Kosovo |  |
| Kuwait | EU868 |
| Kyrgyzstan |  |

#### L

| Country | Frequency Plan |
| --- | --- |
| Laos | AS923_1 |
| Latvia | EU868 |
| Lebanon | EU868 |
| Lesotho | EU433 |
| Liberia |  |
| Libya |  |
| Liechtenstein | EU868 |
| Lithuania | EU868 |
| Luxembourg | EU868 |

#### M

| Country | Frequency Plan |
| --- | --- |
| Macau | AS923_1 |
| North Macedonia | EU868 |
| Madagascar | EU868 |
| Malawi |  |
| Malaysia | AS923_1 |
| Maldives |  |
| Mali | EU433 |
| Malta | EU868 |
| Marshall Islands |  |
| Martinique | EU868 |
| Mauritania | EU868 |
| Mauritius | EU433 |
| Mayotte | EU868 |
| Mexico | US915 |
| Micronesia |  |
| Moldova | EU868 |
| Monaco | EU868 |
| Mongolia |  |
| Montenegro | EU868 |
| Montserrat | AU915 |
| Morocco | EU433 |
| Mozambique |  |
| Myanmar [Burma] | AS923_1 |

#### N

| Country | Frequency Plan |
| --- | --- |
| Namibia | EU868 |
| Nauru |  |
| Nepal |  |
| Netherlands | EU868 |
| Netherlands Antilles |  |
| New Caledonia | EU868 |
| New Zealand | AU915 |
| Nicaragua | AU915 |
| Niger | IN865 |
| Nigeria | EU868 |
| Niue | AU915 |
| Norfolk Island | AU915 |
| North Korea |  |
| Northern Mariana Islands | US915 |
| Norway | EU868 |

#### O

| Country | Frequency Plan |
| --- | --- |
| Oman | EU868 |

#### P

| Country | Frequency Plan |
| --- | --- |
| Pakistan | IN865 |
| Palau |  |
| Palestinian Territories |  |
| Panama | AU915 |
| Papua New Guinea | AU915 |
| Paraguay | AU915 |
| Peru | AU915 |
| Philippines | AS923_3 |
| Pitcairn Islands |  |
| Poland | EU868 |
| Portugal | EU868 |
| Puerto Rico | US915 |

#### Q

| Country | Frequency Plan |
| --- | --- |
| Qatar | EU868 |

#### R

| Country | Frequency Plan |
| --- | --- |
| Réunion | EU868 |
| Romania | EU868 |
| Russia | RU864 |
| Rwanda | EU868 |

#### S

| Country | Frequency Plan |
| --- | --- |
| Saint Helena |  |
| Saint Kitts and Nevis | AU915 |
| Saint Lucia | AU915 |
| Saint Pierre and Miquelon | EU868 |
| Saint Vincent and the Grenadines | AU915 |
| Samoa | EU868 |
| San Marino | EU868 |
| São Tomé and Príncipe |  |
| Saudi Arabia | EU868 |
| Senegal | EU868 |
| Serbia | EU868 |
| Seychelles | EU433 |
| Sierra Leone |  |
| Singapore | AS923_1 |
| Slovakia | EU868 |
| Slovenia | EU868 |
| Solomon Islands | AS923_1 |
| Somalia | EU868 |
| South Africa | EU868 |
| South Georgia and the South Sandwich Islands | EU868 |
| South Korea | KR920 |
| Spain | EU868 |
| Sri Lanka | AS923_1 |
| Sudan |  |
| Suriname | AU915 |
| Svalbard and Jan Mayen | EU868 |
| Sweden | EU868 |
| Switzerland | EU868 |
| Syria | EU868 |

#### T

| Country | Frequency Plan |
| --- | --- |
| Taiwan | AS923_1 |
| Tajikistan |  |
| Tanzania | AS923_1 |
| Thailand | AS923_1 |
| Timor-Leste |  |
| Togo | EU433 |
| Tokelau | AU915 |
| Tonga | AU915 |
| Trinidad and Tobago | AU915 |
| Tunisia | EU868 |
| Turkey | EU868 |
| Turkmenistan |  |
| Turks and Caicos Islands | AU915 |
| Tuvalu |  |

#### U

| Country | Frequency Plan |
| --- | --- |
| U.S. Minor Outlying Islands | US915 |
| U.S. Virgin Islands | US915 |
| Uganda | IN865 |
| Ukraine | EU868 |
| United Arab Emirates | EU868 |
| United Kingdom | EU868 |
| United States | US915 |
| Uruguay | AU915 |
| Uzbekistan | EU433 |

#### V

| Country | Frequency Plan |
| --- | --- |
| Vanuatu | AS923_3 |
| Vatican City | EU868 |
| Venezuela | AS923_1 |
| Vietnam | AS923_2 |

#### W

| Country | Frequency Plan |
| --- | --- |
| Wallis and Futuna | EU868 |
| Western Sahara |  |

#### Y

| Country | Frequency Plan |
| --- | --- |
| Yemen |  |

#### Z

| Country | Frequency Plan |
| --- | --- |
| Zambia | EU868 |
| Zimbabwe | EU433 |

# APPENDIX D

HOTSPOT MANUFACTURERS

- Bobcat Miner 300 https://www.bobcatminer.com/

- SenseCAP M1 https://www.sensecapmx.com/shop/SenseCAP-M1-LoRaWAN-Indoor-Gateway-2G-RAM-US915-1.html

- Dragino HP0D https://www.dragino.com/products/lora-lorawan-gateway/item/195-hp0d.html

- Controllino https://hotspot.controllino.com/product/controllino-hotspot-batch-3c-868-mhz/

- ClodPi https://india-shop.clodpi.io/

- Calchip https://www.calchipconnect.com/

- Finestra https://thefinestra.com/

- Lindox https://www.linxdot.com/store/

- Heltec https://heltec.org/project/ht-m2808/

- Hummingbird https://www.xdt.com/product.html

- MIlesight https://www.milesight-iot.com/

- LongAp https://longap.com/

- MNTD. https://getmntd.com/

- Nebra https://uk.pi-supply.com/collections/helium-hotspot-miners-hnt

- Pisces https://www.piscesiot.com/#/home

- RizingHF https://risinghf.com/product/detail/26

- Syncrob https://syncrob.it/

- FreedomFi(5G) https://freedomfi.com/helium5g/